# Pin Mapping Document – Car Side Mirror Integration

Controller: STM32 NUCLEO-G474RE / G431RB / G491RE
Scope: Mirror Adjustment, Mirror Heating, Turn Signal Indicator

## 1. Power Pins

| Function | STM32 Pin | Description |
| --- | --- | --- |
| Logic Power | 5V / 3.3V | Board supply |
| Ground | GND | Common ground |

## 2. Mirror Adjustment Motor Pins

| Function | STM32 Pin | Description |
| --- | --- | --- |
| Motor X Direction A | D2 | Horizontal direction control |
| Motor X Direction B | D3 | Horizontal direction control |
| Motor X PWM | D5 | Speed control (Timer PWM) |
| Motor Y Direction A | D6 | Vertical direction control |
| Motor Y Direction B | D7 | Vertical direction control |
| Motor Y PWM | D9 | Speed control (Timer PWM) |

## 3. Mirror Heating Pins

| Function | STM32 Pin | Description |
| --- | --- | --- |
| Heater Control | D10 | MOSFET / Relay control |
| Temperature Sensor (Optional) | A0 | ADC input |

## 4. Turn Signal Indicator Pins

| Function | STM32 Pin | Description |
| --- | --- | --- |
| Turn Signal LED | D11 | Blinking LED output |

## 5. Notes

- Motors, heater, and LEDs must be powered from 12V vehicle supply.
- Use external drivers (H-bridge, MOSFETs).
- Do not connect high-power loads directly to STM32 pins.
- Ensure proper grounding and protection circuits.